# Tagging in Jord

# COMMENTS BY VICTOR AGROSKIN

# TECHINVESTLAB.RU

| DESIGN DOCUMENT COVER SHEET |  |
| --- | --- |
| Software Name: | Target Software Version: |
| Requirement Name: Blues Project | Target Release Date: |

| FUNCTIONAL REQUIREMENT REVIEW & APPROVAL |  |
| --- | --- |
| Submitted By: Keith Willshaw | Date: 09 Jan 2013 |
| Approved By: (Dev. Management) Date | Date: |

| REVISIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Revision | Description | Prepared By / Date | Reviewed By / Date | Reviewed By / Date | Approved By / Date |
| 01 | In Progress | K Willshaw 09 Jan 2013 |  |  |  |
| 02 | Updated with reviewer comments | K Willshaw 11 Jan 2013 |  |  |  |

# Table of Contents

1.0	Table of Contents	2

2.0	Associated Documents	3

3.0	Purpose of Document	3

4.0	Definitions	4

4.1	The P&ID Drawing	4

4.2	Selected Fragment	4

5.0	Business Objects	4

5.1	Pumps	5

6.0	Applying the Jord Mapping Methodology	6

6.1	Selecting my Object Of Interest	6

6.2	Does the Class Exist	7

6.3	Declare the Class	7

6.4	Is the Class Related to another Class with its own information	8

6.5	Identification	9

7.0	Applying the Jord Whole Life Individual Method	11

8.0	Conclusions	12

# Associated Documents

The Jord Mapping Methodology

http://www.infowebml.ws/mapping-methodology/index.htm

This is an implementation of Jord Mapping Methodology with a specific template set by Hans Teijgeler. Official Jord Mapping Methodology (not completed to template set level yet) can be found at

https://www.posccaesar.org/attachment/wiki/FiatechJord/JORD-ISO15926-Mapping-Methodology-V2.doc

The Equipment List and Data identified as being key deliverables for a P&ID by the MIMOSA OGI Pilot SIG available at

http://www.mimosa.org/?q=webfm

Under the OGI Pilot Team Utilities >files>Worley Parsons folder

The Jord Endpoint at

http://posccaesar.org/endpoint/#

# Purpose of Document

The document is intended to address the following issues

To model a tag name for an object on Mimosa/OGI Pilot project P&ID usinf Classes, Properties and Templates using the Class methodology outlined in the Jord Mapping Methodology as implemented!

# Definitions

## The P&ID Drawing

The selected drawing is drawing no 11-004 as drawn by Worley Parsons and down loaded from the Mimosa OGI Pilot Repository at

http://www.mimosa.org/?q=webfm

## Selected Fragment

The Selected Fragment is defined in the diagram below. Individual sections of this fragment will be analyzed in detail in later sections

# Business Objects

Business objects typically represent either single plant assets or collections of such assets. An example of a collection of physical assets is an Instrument Loop. When we transfer data from one system to another we need to specify what the Object is, list its properties, relationships and any constraints. The standard being used in the Plant Industry is ISO 15926 which has several parts which help us define the method by which we accomplish this task.

For simplicity we will concentrate initially on a single class of Business Objects namely the Pumps

P-11100A

P-11100B

P-11100C

P-11100D

## Pumps

In ISO 15926 a pump is defined as a physical object that is a driven piece of equipment in which energy is either constantly or periodically added to an amount of pumped liquid in order to increase the pressure required for the process in which the pump is in operation.

The Data modelled in the sample data fragment is as follows:

| EQUIPMENT LIST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equi. No. | Equi. Name | Type | P&ID Name | Const Status | Materials of Constn | Piping Materials Class | Heat Tracing Reqmt | Heat Tracing Temp | Insuln. Purpse | Insuln Thick |
| P-11100A | DEBUTANIZER OVERHEAD PUMP | Horizontal Centrifugal Pump | 11-004 | New |  |  |  |  | N |  |
| P-11100B | DEBUTANIZER OVERHEAD PUMP | Horizontal Centrifugal Pump | 11-004 | New |  |  |  |  | N |  |
| P-11100C | DEBUTANIZER OVERHEAD PUMP | Horizontal Centrifugal Pump | 11-004 | New |  |  |  |  | N |  |
| P-11100D | DEBUTANIZER OVERHEAD PUMP | Horizontal Centrifugal Pump | 11-004 | New |  |  |  |  | N |  |

# Applying the Jord Mapping Methodology

Lets start with the Tag Number as the first thing we need to do is identify which Pump we are dealing with

## Selecting my Object Of Interest

So looking at the JORD decision tree I see the following

I have selected the Pump and my narrative is fairly straightforward. These are the pumps that will be required to pump the process fluid to the Debutanizer receiver

The next decision step is a little trickier as its asking me to make a hard choice about which of the 4 stages I am dealing with, namely

Process Design

Design&Engineering

Procurement&Construction

Operations and Maintenance

While I am primarily concerned with Plant Design and Detailed engineering this is NOT completely divorced from the procurement phase. The deliverables from the design process , specifically the Equipment List, Valve List and Instrument Index are key documents used in the procurement process and its common practise to feed information from the procurement process back to the designer.

One of the key considerations here is a need for change tracking and management to be built in ISO 15926 models. If you are doing modelling for an ongoing design process, where each new data transfer from one system to another can bring new variants of objects and\or properties, your choice is right. But if your goal is one-time transfer to procurement system, some solutions can be simpler, as you had seen working with templates for individuals.

Examples encountered in the past include equipment weights, power consumption and nozzle sizes.

However we will put this aside for now and follow the decision tree to Deal with Part 2 Classes

## Does the Class Exist

My first problems is that I don’t know how to find out if the class exists !

If you’ve only started dealing with Mimosa OGI Pilot facility, you can be sure that classes representing pumps P-11100A-D don’t exist. If you’ve already modelled other datasets from the project – you’ve to check by tags - probable they were already modelled as part of some specification, data sheet, 3D model, whatever. Then their Ur-Classes already have some URIs assigned.

## Declare the Class

Assuming that  the class does not exist I click on the box

“Declare the Class”

However what I see is this

Note_Gen_2

Example of declaration of a Part 2 Class

<owl:Class rdf:ID="Ce269d540-678a-11e1-b86c-0800200c9a66"> <!-- UUID preceeded by a letter (here a C for owl:Class) -->

<rdfs:label>Pump P-101</rdfs:label>

<rdf:type rdf:resource="&dm;ClassOfInanimatePhysicalObject"/>

<rdfs:subClassOf rdf:resource="&rdl;R4598459832"/> <!-- instance of &dm;ClassOfFunctionalObject called "Pump Function" -->

</owl:Class>

NOTE - This is the "Ur-Class" of P-101 and therefore the superclass shall not be more than the applicable instance of the Core ClassOfFunctionalObject. During design and engineering information will be added, changed, and deleted by means of templates, causing the existence of "variants" of this class.

This really isn’t very helpful even to those of us who can read OWL, to the average engineer its Gobbledegook. I need to know HOW to do the following

Check if a Class already exists

Declare one if it does not.

The mapping methodology document offers no way to do this !

Please consider that mapping methodology is designed (is being designed) for your average engineer, but it is not designed to be used entirely by hand, with text editor as a tool.

What are you doing now? You (engineer) are probable sitting together with your specialist software engineer preparing mapping specification to be realized as an adapter software.

Even if not – you are probable looking at the interface of a specialised data editing software (like our .15926 Editor) where simple New command, or drag-n-drop interface, handles creation of a new entity and writes all necessary incantations to the file.

So lets pass on to the next entry in the mapping methodology

## Is the Class Related to another Class with its own information

Not for Tag Number so follow the process to the next decision node

This seems straightforward enough enough so I click Identification

## Identification

Unfortunately I got a list of templates I could use rather than just one so lets pick what seems to be the best fit by reviewing what I as an engineer need to say

Unique Identifier of Thing : basically a GUID (it is usually called “URI” if you are implementing ISO 15926 in Semantic Web world, in RDF/OWL formats)

The Text String I will use for the identifier : P-11100A

A method of defining the Identifier : Tag Number or Tag Identification Code

I dutifully click on identification and using the constraints that this is a Part2 Class not an individual

See what looks to be an appropriate template - ClassifiedIdentificationOfClass

My first reaction is confusion as only 2 of the roles have any entries in the column Remarks & Examples and I don’t have any firm guidance on how to proceed

You’ve decided to use a template set with Ur-Class roles, designed to handle life-cycle data modelling and class definition variability as its part. The Ur-Class methodology as described on

http://15926.org/publications/general-discussions/class-model/index.htm

is particularly unsuitable for implementation by hand. Mapping to this set is definitely not for an average engineer, it’s usage requires understanding of some ontological conventions and their set theory implications.

I don’t know yet what to do with it. An obvious solution is to hide all these difficulties under the hood of a mapping adapter. And adapter for these templates should have complex internal logic, for example we in .15926 project are using fully functional programming language enhanced with 15926-specific constructs to do the job.

Development of such an adapter requires direct collaboration between a plant engineer, data modeller and software engineer. Yes, 3 persons, the number can be reduced to 2, and you are fortunate if you can find one professional in three domains (I was unable so far).

Looking at the Lowered Template roles in detail I see

Role 1 : I don’t know how to get this value if it already exists or how to create it if it doesn’t.

This is exactly the URI of a class you’ve created at 6.3 above.

Role 2 : OK I know how to get an identifier for this

No, you probable don’t. This is a “temporary” class, to record “temporary” nature of tag assignment (yes, in Ur-Class methodology even tag assignment is temporary and can in theory be changed at any moment during life-cycle). This URI should be generated by software and remembered for composition of an Object Information Model for P-11100….

Role 3 : OK the value of the Identifier “P-11100A” fits here

Role 4 : In the RDL we used TagIdentificationCode which was defined as

‘An identification code for a 'functional_physical_object'.

Unfortunately this all falls apart when I look for Tag Identification Code in the Jord Endpoint because I find its been flagged as RETIRED and there’s an instruction to use 'tagged object code string class'

When I look at this I find it’s NOT a ClassofClassOfIdentification but is defined as RepresentationForm which in part 2 is defined as

A <representation_form> is a <class_of_class_of_information_representation> that distinguishes the form of representation.
EXAMPLE Hexadecimal, text, script, symbol, picture, diagram, semaphore, Morse code, music score, MIDI file format, and XML can each be represented by instances of <representation_form>.

So we take another look and I find something called TAGGED OBJECT CODE ASSIGNMENT CLASS

Now this is a ClassofClassOfIdentification but frustratingly has no notes or description.

Yes, it looks like a whole complex of mistakes was corrected in RDL without proper notification of users. But these were mistakes indeed, as relationships should be named with verbs of verb-like constructs, and classes of individuals – with nouns.

“... CODE” is not a good name for CoCoR!

The template in question requires classification of relationship. There is a discussion – whether it is theoretically enough to classify identification strings only, but identification of strings is certainly suboptimal from software realisation POV.

So the best I can do is guess that we need to create a member of ClassOfClassOfIdentification, probably in the Project RDL, such as OGI PILOT PROJECT TAG ASSIGNMENT CLASS  Yes, but please abide by naming rules and do not forget to specialize your new RD entity  from OBJECT CODE ASSIGNMENT CLASS you’ve discovered in JORD RDL.

SO we end up with the following

| Role No | Role Name | Role Type | Typical Value |
| --- | --- | --- | --- |
| 1 | hasUrClass | Class | Created at 6.3 above |
| 2 | hasIdentifified | Class | Generated by the software |
| 3 | valIdentifier | ClassOfExpressInformationRepresentation | “P-11100A” |
| 4 | hasIdentificationType | ClassofClassOfIdentification | OGI PILOT PROJECT TAG ASSIGNMENT CLASS (see Notes 1 & 2) |

Notes

1) I assume that an example would an entry in the project RDL of entity type ClassOfClassOfIdentification OGI PILOT PROJECT TAG ASSIGNMENT CLASS – is this correct ?

2) This appears very useful as it gives me a way of properly classifying alternative tags for a functional object such as OldTagName, BechtelTagName etc Exactly. Just do not forget to form correct names and also specialize them from JORD RDL class.

# Applying the Jord Whole Life Individual Method

I now turned my attention to the ‘simpler’ WholeLifeIndividual

Following the same steps as for the Class Model I end up with the template - Classified Identification of Individual

As with the Class Model the entry in Remarks & Example is a little cryptic so once again I have to guess.

The Current iRING mapping uses the initital set template ClassifiedIdentification and looks like this

| Role No | Role Name | Role Type | Typical Value |
| --- | --- | --- | --- |
| 1 | hasIdentificationType | ClassOfClassOfIdentification | TAG IDENTIFICATION CODE |
| 2 | hasObject | PossibleIndvidual | Unique identifier of Thing |
| 3 | valIdentifier | Express String | P-11100A |

This has some serious problems as TAG IDENTIFICATION CODE is the wrong  entity type being a DocumentDefinition and has been Retired and as before there’s an instruction to use 'tagged object code string class'

The same source of problems, the same solution as above. The same OGI PILOT PROJECT TAG ASSIGNMENT CLASS should be used, as identification assignment for class and for individual are the same.

My assumption is that with the JORD Template ClassifiedIdentificationOfIndividual I would seem something like

| Role No | Role Name | Role Type | Typical Value |
| --- | --- | --- | --- |
| 1 | hasIdentifified | PossibleIndvidual | Unique identifier of Pump |
| 2 | valIdentifier | RepresentationForm | P-11100A |
| 3 | hasIdentificationType | ClassOfClassOfIdentification | OGI PILOT PROJECT TAG ASSIGNMENT CLASS (see Notes 1 & 2) |

Notes

There are no satisfactory examples of ClassOfIdentification in the RDL (you don’t need it) however there is a ClassOfClassOfIndentification called TAGGED OBJECT NAME ASSIGNMENT CLASS. Is this correct ? No, identification of individual is classified with the same OGI PILOT PROJECT TAG ASSIGNMENT CLASS which is ClassOfClassOfIdentification.

If so this allows more than one tag numbering schema to be around and allows support for entries such as ‘Old Tag Number’ ,  ‘Vendor Tag Number’  etc

# Conclusions

We need to clear this up ASAP, Tagging of Plant Items is the essential pre-requisite for any data exchange and there are multiple industry exchange projects being fast tracked. Examples include the Mimosa OGI Pilot and DECHEMA initiatives. Additionally people are already using the modelling methodology rolled out in the iRING initiative so time is of the essence here.